Abstract

In this document you find a detailed guidance about how to build a self-driving car architecture with Robot Operating System

Building Self-driving car architecture 
with 
robot operating system

ISsoft Insights 2019 – Workshop Guide

Alexey Sas, Oleg Karasik

alexey.sas@coherentsolutions.com, oleg.karasik@coherentsolutions.com

# Introduction

Welcome to ISsoft Insights 2019 workshop “Building Self-Driving Car architecture with Robot Operating System”.

This “tiny” document contains step-by-step guidance about how to develop a simple yet working self-driving car application.

| To make workshop more interactive we have decided to remove all overhead related to initial environment configuration and setup. For each attendee we have created a dedicated virtual environment in Azure with preconfigured software. All required materials including source code, detailed information about installed software and its configuration is available here |  |  |
| --- | --- | --- |

Stay focused and have fun! Let’s go!

# Document legend

Besides text you will see multiple types of blocks in this document:

Command Snippet

This block contains a command prompt command (or several commands, each in a separate row). If not mentioned explicitly you should copy this command to the last used command prompt – regardless whether this is Anaconda Prompt, Windows Command Prompt or Bash Shell. In case of multiple commands – each command should be copied and executed in order of appearance.

COMMAND SNIPPET

<command>

Code Snippet

This block contains file name, action, relative position and piece of code block to insert. If information is omitted, then specified content of the file should be replaced with code block.

CODE SNIPPET

<file> (action, relative position)

<code>

This block also has a second variation used to express replacement of one code block with another.

CODE SNIPPET

<file> (replace, relative position)

<original code>

with

<replacement code>

Attention

This block contains important information or comment. While block content does not directly affect the step, where it is mentioned, its content is worth reading.

|  | <important notice or warning> |
| --- | --- |

# Phase 1: Preparations

## Connect to remote environment

The first step in our workshop is to make connection to remote virtual environment. You can do this from both Windows or Mac.

| On Windows |  | Place connection details here |
| --- | --- | --- |
| Use Remote Desktop Connection application to connect to remote environment. |  |  |
| On Mac |  |  |
| Use Microsoft Remote Desktop 10 application to connect to remote environment |  |  |

When connected you should see standard Windows 10 desktop.

## Start background software

While all software is preinstalled, it is not set to run automatically on system startup. Why? That is easy – running this software is a part of the development process we would like to get acquainted with.

### Start XServer

The first element we need to launch is the XServer. This application would create a XServer Display used by Linux subsystem to render graphics.

| On the top left corner of Windows desktop there is a shortcut called “XLaunch” |  |  |
| --- | --- | --- |

Double click on it to run.

DO NOT change any application settings. Just click: Next → Next → Next → Finish.

### Start CARLA server

CARLA is a complex self-driving car simulator based on Unreal Engine. In general, simulator consists of two parts: a client and a server. The server part is the “world” with building, traffic lights, pedestrians, cars, …, etc.

During the workshop we will use one of the default “worlds” – a city map created by CARLA team.

To start CARLA server, we need to open a new Anaconda Prompt. In the search bar (on the bottom left) type “anaconda” and open “Anaconda Prompt” from best matches.

In the opened command prompt type:

COMMAND SNIPPET

cd C:\Workshop\carla-server

- This sets current directory of anaconda prompt to: С:\Workshop\carla-server

Launch the CARLA Server using the following command:

COMMAND SNIPPET

CarlaUE4.exe -windowed -ResX=640 -ResY=480

- This launches CARLA server in windowed mode with windows size set to 640x480 pixels.

| Application could take a while to start but when it is done you should see a picture like this: |  |
| --- | --- |

|  | When CARLA Server is started it will capture your cursor inside it. Use ALT+TAB to switch out and minimize the window. DO NOT CLOSE CARLA SERVER WINDOW |
| --- | --- |

### Start CARLA – ROS bridge

To enable interactions between ROS and CARLA server we need to establish a “bridge” between them. CARLA ROS bridge is a ROS node created with knowledge of CARLA server API. Bridge defines a set of messages and topics other nodes can use to send commands to CARLA server and receive information about what is happening in the world.

To start the bridge, we need to open a new bash command prompt. In the search bar type “bash” and open “Bash” from best matches.

In the opened command prompt type:

COMMAND SNIPPET

roslaunch carla_ros_bridge client.launch

- This launches ROS master process (covered later) and carla_ros_bridge node.

|  | DO NOT close bash prompt - minimize it. |
| --- | --- |

### Start Carla Client

To control a vehicle, we need… a vehicle. We will not use custom code to create a vehicle, but instead of this we would use client created by CARLA team. The client creates a random vehicle, initializes its sensors and allows you to manually control it by using keyboard.

To start client, we need to open new Anaconda Prompt. In the opened command prompt type:

COMMAND SNIPPET

cd C:\Workshop\carla-client

python manual_control.py --res 640x480

- This launches manual_control.py python script in a window of 640x480 pixels.

| When client is launched you should see a picture from car’s camera. You can check anaconda prompt for instruction of how to control the vehicle and play with it for a while. When you are done close the window and relaunch the client. DO NOT close newly opened window - minimize it. |  |
| --- | --- |

## Creating ROS package

ROS package contains code, scripts, message and files required to build and make nodes from this package discoverable. We would start from the simplest scenario – an empty package.

Open a new bash prompt and type the following commands:

COMMAND SNIPPET

mkdir -p /mnt/c/Workshop/sdc_workspace/catkin_ws/src

cd /mnt/c/Workshop/sdc_workspace/catkin_ws

- This creates /mnt/c/Workshop/sdc_workspace/catkin_ws/src and sets bash prompt current directory to /mnt/c/Workshop/sdc_workspace/catkin_ws.

|  | In Windows subsystem for Linux path /mnt/* is mapped to appropriate logical drive i.e. /mnt/c → C:\, /mnt/d → D:\. |
| --- | --- |

The directories we have just created are required to initialize catkin workspace. You can think of Catkin as of build infrastructure used to create ROS packages – very similar to angular-cli for Angular.

This command will initialize a new catkin workspace:

COMMAND SNIPPET

catkin_make

Package is also created using Catkin:

COMMAND SNIPPET

cd src

catkin_create_pkg sdc_package sensor_msgs cv_bridge std_msgs rospy

- This invokes catkin_create_pkg macro which creates a new package named sdc_package which depends on: sensor_msgs, cv_bridge, std_msgs and rospy packages.

| Using File Explorer navigate to C:\Workshop\sdc_workspace\catkin_ws\src\sdc_package and open it in Visual Studio Code. |  |
| --- | --- |

# Phase 2: Programming ROS nodes

## Create “Hello World” node

The first thing to begin with is always a “Hello World” something. In case of ROS – “Hello World” node.

| In Visual Studio Code create new scripts directory inside sdc_package directory. Then inside it, create hello.sh file. |  |
| --- | --- |

Inside the hello.sh type the following code:

CODE SNIPPET

scripts/hello.sh

#!/bin/bash echo Hello World!

- Line #1 instructs shell to execute this as bash shell script.

- Line #2 uses echo application to print “Hello World!” into terminal.

We already have ROS master process online (we have launched it when started CARLA ROS bridge) so to run hello.sh node all we need is to open the bash command prompt we have used to create package and execute the following commands:

COMMAND SNIPPET

cd ..

source devel/setup.bash

rosrun sdc_package hello.sh

- #1 – changes current directory to /mnt/c/Workshop/sdc_workspace/catkin_ws.

- #2 – loads workspace related environment variables into shell context.

- #3 – runs hello.sh node from sdc_package.

As the result you should see “Hello World” output in terminal.

|  | Future reference You can start ROS master process using roscore command in bash command prompt. |
| --- | --- |

Running nodes like this could be fine when you have one or two nodes but when there are multiple nodes and these nodes have its own configuration running all this setup manually could be problematic.

This problem is solved by the infrastructure. It allows you to define a .launch with list of nodes, parameters and references to another .launch files and launch this with single command.

| In Visual Studio Code create new launch directory inside sdc_package directory. Then inside it, create hello.launch file. |  |
| --- | --- |

Inside the hello.launch type the following code:

CODE SNIPPET

launch/hello.launch

<?xml version="1.0"?> <launch> <!-- pkg: defines package name type: defines file to launch name: defines unique node name output: defines application output destination --> <node pkg="sdc_package" type="hello.sh" name="hello_node" output="screen"> </node> </launch>

Open the bash command prompt we have previously used to run hello.sh and type the following command:

COMMAND SNIPPET

roslaunch sdc_package hello.launch

- This starts roslaunch server to connect to ROS master process and run all ROS nodes included in .launch file.

As the result you should see “Hello World” output in terminal.

## Create camera node

We have already created node to execute the .sh script but it had no logic and literally did nothing except printing “Hello World!”. It is time to write something that has logic behind it. In our case we write a node to capture, process and display images from car’s camera.

Open File Explorer in C:\Workshop\project\templates directory and copy camera.py and utilities.py to sdc_package\scripts directory.

Open scripts\camera.py in Visual Studio Code and configure file to be executed as python script.

CODE SNIPPET

scripts/camera.py (insert after, TODO: 1)

#!/usr/bin/env python

All ROS nodes should have a unique name. In python scripts this is done by using the following code:

CODE SNIPPET

scripts/camera.py (insert after, TODO: 2)

rospy.init_node('camera')

Now we need to do one of the essential things in ROS – we would subscribe to a topic. In our case this would be one of the topics created by CARLA ROS bridge. This topic supplies images from car’s front camera.

CODE SNIPPET

scripts/camera.py (insert after, TODO: 3)

rospy.Subscriber( '/carla/ego_vehicle/camera/rgb/front/image_color', Image, self.process_image)

- This subscribes self.process_image to run every time '/carla/ego_vehicle/camera/rgb/front/image_color' topic has new Image message.

|  | The ‘Image’ message format description can be found here or by QR Code. |
| --- | --- |

The last edit in camera.py is to implement the callback:

CODE SNIPPET

scripts/camera.py (insert after, TODO: 4)

def process_image(self, image): self.got_image = True self.image = self.bridge.imgmsg_to_cv2(image, "bgr8")

We have done with the script and now we need to create camera.launch in launch directory.

In launch directory create camera.launch with the following content:

CODE SNIPPET

launch/camera.launch

<?xml version="1.0"?> <launch> <node pkg="sdc_package" type="camera.py" name="camera" output="screen"> </node> </launch>

Open the bash command prompt we have previously used to run hello.launch and type the following command:

COMMAND SNIPPET

roslaunch sdc_package camera.launch

As the result you should see the picture from car’s front camera:

| View from manual_control.py | View from camera.launch |
| --- | --- |

Sounds boring? Then let’s add more fun.

Do the following change in camera.py and relaunch camera node:

CODE SNIPPET

scripts/camera.py (uncomment, TODO: 5)

# frame = pipline(frame)

Now it looks cool! The pipeline function we uncommented filters image to make it useful for line detection.

| View from manual_control.py | View from camera.launch |
| --- | --- |

## Create controller node

Capturing camera images allows us to see what is happening with our vehicle. In more realistic scenarios we could make further image processing to extract more data, but this is more about mathematics and algorithms rather than self-driving cars.

Our next step is to start controlling the vehicle. This can be done by publishing a CarlaVehicleControl message to ‘/carla/ego_vehicle/vehicle_control_cmd’ topic.

Create a new msg directory in sdc_packages directory. Inside msg directory create CarlaVehicleControl.msg.

Inside CarlaVehicleControl.msg type the following code:

CODE SNIPPET

msg/CarlaVehicleControl.msg

Header header float32 throttle float32 steer float32 brake bool hand_brake bool reverse

Simply adding a message into a package is not enough. The reason behind it is very simple – the message is the binary contract. So, to make it work we need to compile all messages included into our package. Package build is defined in two files: package.xml and CMakeList.xml.

Define sdc_package dependency on message_generation and message_runtime packages:

CODE SNIPPET

package.xml (insert at, line: 56, 65)

<build_depend>message_generation</build_depend>

<exec_depend>message_runtime</exec_depend>

Include message_generation package into list of packages to find on build (find_package):

CODE SNIPPET

CMakeList.txt (insert at, line: 15)

message_generation

Include message_runtime into dependencies list of sdc_package library:

CODE SNIPPET

CMakeList.txt (replace, lines: 107-112)

catkin_package( # INCLUDE_DIRS include # LIBRARIES sdc_package # CATKIN_DEPENDS cv_bridge rospy sensor_msgs std_msgs # DEPENDS system_lib )

with

catkin_package( LIBRARIES sdc_package CATKIN_DEPENDS cv_bridge rospy sensor_msgs std_msgs message_runtime )

Include CarlaVehicleControl.msg into list of messages to compile for our package:

CODE SNIPPET

CMakeList.txt (replace, lines: 52-56)

# add_message_files( # FILES # Message1.msg # Message2.msg # )

with

add_message_files( FILES CarlaVehicleControl.msg )

Turn on message generation build step:

CODE SNIPPET

CMakeList.txt (replace, lines: 72-75)

# generate_messages( # DEPENDENCIES # sensor_msgs# std_msgs # )

with

generate_messages( DEPENDENCIES sensor_msgs std_msgs )

Open the bash command prompt we have previously used to run camera.launch and type the following commands:

COMMAND SNIPPET

catkin_make

source devel/setup.bash

|  | Probably you’ve noticed that bash prompt you’ve opened is busy with ROS running our camera.launch. To stop ROS, you should use CTRL+C combination – this would trigger ROS termination. |
| --- | --- |

Messages compilation can be verified with the following command:

COMMAND SNIPPET

rosmsg show sdc/CarlaVehicleControl.msg

Copy simple_controller.py from templates directory to sdc_package\scripts directory and open it in Visual Studio Code.

The simple_controller.py is a template we would later tweak to implement final controller but no worries it is prepared for simple implementation too.

Include CarlaVehicleControl message:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO 1.1)

from sdc_package.msg import CarlaVehicleControl

Initialize node name:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO 1.2)

rospy.init_node('simple_controller')

Create publisher to publish messages to '/carla/ego_vehicle/vehicle_control_cmd' topic:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 1.3)

self.control_publisher = rospy.Publisher( '/carla/ego_vehicle/vehicle_control_cmd', CarlaVehicleControl, queue_size = 1 )

Enable basic control routine:

CODE SNIPPET

scripts/simple_controller.py (uncomment, TODO: 1.4)

#throttle_output, steer_output, brake_output = self.basic_control()

Prepare and publish CarlaVehicleControl message:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 1.5)

message = CarlaVehicleControl( None, throttle_output, steer_output, brake_output, False, False) self.control_publisher.publish(message)

In launch directory create simple_controller.launch with the following content:

CODE SNIPPET

launch/simple_controller.launch

<?xml version="1.0"?> <launch> <node pkg="sdc_package" type="simple_controller.py" name="simple_controller" output="screen"> </node> </launch>

Running simple_controller node without visualization does not make much sense. That is why it is time to create a launch file of launch files!

Create all.launch file in launch directory with the following content:

CODE SNIPPET

launch/all.launch

<?xml version="1.0"?> <launch> <include file="$(find sdc_package)/launch/simple_controller.launch" /> <include file="$(find sdc_package)/launch/camera.launch" /> </launch>

Open the bash command prompt we have previously used to run catkin_launch and launch all.launch:

COMMAND SNIPPET

roslaunch sdc_package all.launch

As the result you should see how your vehicle is slowly moving forward.

# Phase 3: Complete self-driving car architecture

Real self-driving car controller requires minimum two sources of information. The first one is the mission route and the second one is a close-range local route. These two sources of information would be implemented using two additional nodes: mission planner and local planner.

## Defining data and messages

Create data directory in sdc_package directory and copy waypoints.csv from C:\Workshop\project\data directory.

In msg directory create new messages – BaseWaypoint.msg, Path.msg, Waypoint.msg and LocalPath.msg with the following content:

CODE SNIPPET

msg/BaseWaypoint.msg

float32 x float32 y

msg/Path.msg

BaseWaypoint[] waypoints

msg/Waypoint.msg

float32 x float32 y float32 v

msg/LocalPath.msg

Waypoint[] waypoints

In CMakeLists.txt add these messages to the end of add_message_files() block:

CODE SNIPPET

CMakeLists.txt (insert at, line: 55)

BaseWaypoint.msg Path.msg Waypoint.msg LocalPath.msg

Open the bash command prompt we have previously used to run all.launch and type the following command:

COMMAND SNIPPET

catkin_make

source devel/setup.bash

## Create mission planner node

Copy mission_planner.py from templates directory to sdc_package/scripts directory and open it in Visual Studio Code.

Include BaseWaypoint and Path messages:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 1)

from sdc_package.msg import BaseWaypoint, Path

Initialize node name:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 2)

rospy.init_node('mission_planner')

We create publisher that will be used to publish Path message to ‘/planner/mission_waypoints’ topic:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 3)

self.waypoints_publisher = rospy.Publisher( '/planner/mission_waypoints', Path, queue_size = 1)

We will not retrieve mission waypoint from real source (GPS, road detection, …, etc.). Instead of this we will load and publish a predefined set of endpoints.

Firstly, we should obtain a path to waypoints.csv data file:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 4)

waypoints_file_path = rospy.get_param('~waypoints_path')

Secondly, we will load waypoints by parsing input CSV into array of BaseWaypoint objects by calling self.load_waypoints function:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 5)

waypoints = self.load_waypoints(waypoints_file_path)

Thirdly, we will publish loaded waypoints by calling self.publish_waypoints function:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 6)

self.publish_waypoints(waypoints)

Fourthly, we will put our node into empty loop by calling rospy.spin() function:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 7)

rospy.spin()

Now we need to tweak self.load_waypoints and self.publish_waypoints functions.

In self.load_waypoints we need to implement CSV parsing logic (which is quite simple):

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 8)

waypoint = BaseWaypoint() waypoint.x = float(row[0]) waypoint.y = -float(row[1]) waypoints.append(waypoint)

In self.publish_waypoints we need to create new Path object from input array of BaseWaypoint objects:

CODE SNIPPET

scripts/mission_planner.py (insert after, TODO: 9)

path = Path(waypoints)

The last thing left is to create a mission_planner.launch file and include it into all.launch. In launch directory create mission_planner.launch with the following content:

CODE SNIPPET

launch/mission_planner.launch

<?xml version="1.0"?> <launch> <node pkg="sdc_package" type="mission_planner.py" name="mission_planner" output="screen"> <param name="waypoints_path" value="$(find sdc_package)/data/waypoints.csv" /> </node> </launch>

In all.launch include mission_planner.launch:

CODE SNIPPET

launch/all.launch (insert at, line: 3)

<include file="$(find sdc_package)/launch/mission_planner.launch" />

## Create local planner node

Copy local_planner.py from templates directory to sdc_package directory and open it in Visual Studio Code.

Include BaseWaypoint, Waypoint, Path and LocalPath messages:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 1)

from sdc_package.msg import BaseWaypoint, Waypoint, Path, LocalPath

Initialize node name:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 2)

rospy.init_node('local_planner')

Subscribe on ‘/planner/mission_waypoints’ topic to receive mission path:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 3)

rospy.Subscriber( '/planner/mission_waypoints', Path, self.init_mission)

Now we need to tweak self.process_position, self.plan_path, self.get_car_closest_waypoint_idx and self.prepare_local_path functions.

The self.process_position function is a callback function registered to receive Odometry messages. Odometry is a standard ROS message type that contains information about current vehicle position, orientation, speed, …, etc.

In this guide we are interested in three components:

- odometry.pose.pose.position – to get current vehicle position

- odometry.pose.pose.orientation – to get current vehicle orientation

- odometry.twist.twist.linear – to get current vehicle speed

|  | The ‘Odometry’ message format description can be found here or by QR Code. |
| --- | --- |

In self.process_position function store position component of Odometry message in self.position:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 4)

self.position = position.pose

In self.plan_path function we need to publish LocalPath create by self.prepare_local_path:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 5)

self.local_publisher.publish(local_path)

In self.get_car_closest_waypoint_idx function we need to find index of closest waypoint to current vehicle position:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 6)

closest_idx = self.waypoint_tree.query([x, y], 1)[1]

Now we have closest waypoint index and can get waypoints subset from the stored waypoints started from this index.

Not exactly true, if we have closest waypoint which is already behind the vehicle we cause vehicle to make a 180-degree turn. So, we can add additional check to see if closest waypoint behind and if so – take next waypoint.

From math the easiest way to check this is to use sign of the dot product of vectors.

|  | Graphical explanation of usage of Sign of Dot Product to make sure A and B facing similar direction can be found here or by QR Code. |
| --- | --- |

In total we need:

- Get next point.

- Calculate vectors based on closest waypoint and next waypoint.

- Check if position and next waypoint are placed in one direction from closest waypoint. If so – select next waypoint as our reference waypoint for trajectory as closest waypoint is already behind.

- Compare dot value with zero and select appropriate index.

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 7)

closest_waypoint = self.waypoints_xy[closest_idx] next_waypoint = self.waypoints_xy[closest_idx + 1] # convert waypoints to numpy vectors closest_as_vector = np.array(closest_waypoint) next_as_vector = np.array(next_waypoint) position_vector = np.array([x, y]) # find vectors of interest v1 = next_as_vector - closest_as_vector v2 = position_vector - closest_as_vector # calculate dot product val = np.dot(v1, v2) if val > 0: closest_idx = closest_idx + 1

In self.prepare_local_path function we need create LocalPath object by adding speed component to BaseWaypoint objects:

CODE SNIPPET

scripts/local_planner.py (insert after, TODO: 8)

result_waypoints = [] for base_waypoint in base_waypoints: waypoint = Waypoint() waypoint.x = base_waypoint.x waypoint.y = base_waypoint.y waypoint.v = PLANNED_SPEED result_waypoints.append(waypoint) path = LocalPath() path.waypoints = result_waypoints

The last thing left is to create a local_planner.launch file and include it into all.launch.

In launch directory create local_planner.launch with the following content:

CODE SNIPPET

launch/local_planner.launch

<?xml version="1.0"?> <launch> <node pkg="sdc_package" type="local_planner.py" name="local_planner" output="screen"> </node> </launch>

In all.launch include mission_planner.launch:

CODE SNIPPET

launch/all.launch (insert at, line: 3)

<include file="$(find sdc_package)/launch/local_planner.launch" />

## Tweaking controller node

The last part of this guide is about tweaking previously written controller to drive the vehicle down mission path.

Copy pid.py and stanley.py from templates directory to sdc_package\scripts directory.

Open simple_controller.py in Visual Studio Code.

Uncomment imports of PID and Stanley controllers:

CODE SNIPPET

scripts/simple_controller.py (uncomment, lines: 5,9-10)

#from pid import PID #from stanley import Stanley

Import LocalPath and Waypoint messages:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 2.1)

from sdc_package.msg import LocalPath, Waypoint

Initialize PID and Stanley controllers by calling self.init_controllers function:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 2.2)

self.init_controllers()

Subscribe to '/planner/local_waypoints' to receive LocalPath:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 2.3)

rospy.Subscriber('/planner/local_waypoints', LocalPath, self.process_waypoints)

Subscribe to '/carla/ego_vehicle/odometry' to receive Odometry:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 2.4)

rospy.Subscriber('/carla/ego_vehicle/odometry', Odometry, self.process_position)

Uncomment call self.wait_initialization() to make sure we will wait for LocalPath and Odometry to arrive:

CODE SNIPPET

scripts/simple_controller.py (uncomment, TODO: 2.5)

#self.wait_initialization()

Now we need to tweak self.process_position, self.process_waypoints, self.loop and self.control functions.

The self.process_position function is a callback to receive Odometry messages. Inside this callback we need to store current vehicle position, velocity and heading direction. Current position is retrieved from the Odometry message itself. Heading direction is received using quaternion. We need to get Euler yaw angle from quaternion, so we can use this angle in Stanley controller (implemented in self.get_yaw_last_position function):

| CODE SNIPPET |  |
| --- | --- |
| scripts/simple_controller.py (replace after, TODO: 2.6) |  |
| self.x = None self.y = None |  |
| self.yaw = None |  |
| with |  |
| self.x = self.position.pose.position.x self.y = self.position.pose.position.y |  |
| self.yaw = self.get_yaw_last_position() |  |
|  | You can find a brief introduction to quaternion and Euler yaw angle here and here or by QR Codes. Quaternion; Euler Yaw Angle. |

Additionally, we have a separate x, y velocity components in the Odometry message. We can use the Pythagorean Theorem to calculate full velocity.

CODE SNIPPET

scripts/simple_controller.py (replace after, TODO: 2.7)

self.v = None

with

self.v = math.sqrt(v_x * v_x + v_y * v_y)

|  | You can find Pythagorean Theorem description here or by QR Code. |
| --- | --- |

The self.process_waypoints function is a callback to receive LocalPath message. Inside this function we need to store waypoints we received from local planner:

CODE SNIPPET

scripts/simple_controller.py (insert after, TODO: 2.8)

self.waypoints = path.waypoints

Inside self.loop function we need to replace throttle, steer and brake parameters received from self.basic_control function with parameters received from self.control function:

CODE SNIPPET

scripts/simple_controller.py (uncomment, TODO: 2.9)

#throttle_output, steer_output, brake_output = self.control(dt)

The controller is tweaked and as it is already linked to all.launch file there is nothing left!

# Phase 4: Test Drive

We have implemented and tweaked everything we planned to and now it is time to test this beast!

Close and Reopen CARLA client.

Open the bash command prompt we have previously used to run all.launch and type the following command:

COMMAND SNIPPET

roslaunch sdc_package all.launch

Enjoy!

# Conclusion

We have explored how to build a self-driving car architecture with Robot Operating System and CARLA self-driving car simulator.

While most of the time we were making “Find and Replace” operations rather than actual coding you still have the opportunity to explore all the mathematics behind this process by exploring links mentioned during the guide and extra information in Extra section.

Hope you have enjoyed the journey!

# Extra

| # | Title (Hyperlink) | QR Code |
| --- | --- | --- |
| 1 | Workshop materials |  |
| 2 | Automatic Steering Methods for Autonomous Automobile Path Tracking |  |
| 3 | Graphical explanation of usage of Sign of Dot Product to make sure A and B facing similar direction |  |
| 4 | Quaternion |  |
| 5 | Euler Yaw Angle |  |
| 6 | Pythagorean Theorem |  |